# BYE LAWS

Made by the North Tyneside Council in pursuance of Section 15 of the Local Government (Miscellaneous Provisions) Act 1982 relating to EAR PIERCING AND ELECTROLYSIS.

1.	For the purpose of securing the cleanliness of the premises and fittings therein a proprietor shall ensure that:-

- All internal walls, doors, windows, partitions floors and floor coverings and ceilings in any part of the premises used by clients and operators are kept clean and in such good repair as to enable them to be cleaned effectively;

- All waste material, and other litter, arising from the treatment, is placed in suitable covered receptacles, which are washable and leakproof, or use a leakproof liner bag.  The receptacles shall be emptied, or the bags changed, at least once every working day, or more frequent as necessary, and the material disposed of safely.  Where liners are not used, the receptacles shall be cleaned.

- All needles used in treatment are placed after use in separate covered and leakproof, re-useable boxes designed for the purpose.  When re-useable boxes are used they shall be emptied at least once every working day or more frequently as necessary and the contents disposed of safely or sterilised for re-use as appropriate. The box shall then be sterilised.  Where needle boxes are used, they shall be disposed of safely at suitable intervals.

- All furniture and fittings in the premises are kept clean and in such good repair as to enable them to be cleaned effectively.

- All tables, couches and seats used by clients in the treatment area and any surface on which the items specified in (b) below are placed immediately prior to treatment, have a smooth impervious surface which is wiped down at least daily with a suitable disinfectant;

- Where tables or couches are used, they shall be covered by a disposable paper sheet which shall be changed for each client;

- A notice or notices reading ’No Smoking’ are prominently displayed within the treatment area.

2.	For the purpose of securing the cleansing and, so far as is appropriate, the sterilisation of instruments, materials and equipment used in connection with the treatment:-

- An operator shall ensure that, before use in connection with treatment, any gown, wrap or other protective clothing, paper or other covering, towel, cloth or other such articles used in the treatment –

i.	is clean and in good repair, and so far as is appropriate, is sterile

- has not been previously used in connection with any other client unless it consists of a material which can be and has been adequately cleaned and so far as is appropriate sterilised.

- An operator shall ensure that any needle , metal instrument, or other item of equipment used in treatment or for handling instruments and needles used in treatment , is in a sterile condition and kept sterile until it is used.

- A proprietor shall provide:-

- adequate facilities and equipment for the purpose of sterilisation(unless pre-sterilised items are used) and of cleansing, as required in pursuance of these Byelaws;

- sufficient and safe gas points and or electrical socket outlets to enable compliance with these Byelaws;

- an adequate constant supply of clean hot and cold water readily available at all times on the premises;

- adequate storage for all items mentioned in Bye law 2(a) and (b)  above; so that those items shall be properly stored in a clean and suitable place so as to avoid, as so far as possible, the risk of contamination.